# SOP

EPSON F2270 DTG/DTF FABRIC PRINTER

OPERATION MANUAL

Training is required before using this equipment. Reservations are required to use this equipment.

# PRINTING PROCESSES

## DTG

Direct To Garment printing can be done on any fabric that is 70% or more cotton. It can be used to print on shirts, jackets, sweatshirts and fabric; it cannot be used to print on denim, sleeves, hats, or pants. The printer will first lay down a base layer of white, then print the colors on top of that to ensure they stand out against the garment. Once cured, the printed design can withstand being washed and dried.

## DTF

Direct To Film printing is used to transfer designs onto fabrics. It can be used on fabrics that cannot be printed on via DTG such as denim, 70% or less cotton fabric, and hats. This is done by printing directly onto a film rather than the garment. Once the film is cured, it is laid on the fabric and cured again to ensure it fuses with the fabric. Garments with DTF designs can withstand being washed and dried when turned inside out.

## DIGITAL FACTORY

This software is used to import images to be printed. It allows you to adjust the size, placement, and rotation of your image to format for printing. You can print multiple images in one go by nesting them on the page.

## GARMENT CREATOR

This software is to be used for PRICING ONLY. Once you upload your design, this program will tell how many mL of ink will be used to print the design. Take a picture or write down how many mL of ink will be used and the cost.

# MATERIAL LIMITATIONS

- DTG can only be done on fabrics that are 70% or more cotton.

*Denim cannot be printed on.

- DTF can be used on fabrics that cannot be printed on such as denim, fabrics made of less than 70% cotton, and hats.

## PRETREATING

- For colored fabrics, the saturation of the hue will determine whether you need to pretreat the fabric before printing on it.

- Colors above the line should not need to be pretreated, colors below the line would have best results if pretreated first.

*You do not need to pretreat fabrics for DTF.

- Turn on the Master switch on the right side of the Lawson pretreatment machine.

*Do not adjust the spray speed knob

- Turn on the heat press. The switch is located on the underside of the swing arm and above the heat platen.

- DO NOT adjust any settings on the heat press- it will beep when at full heat.

- The heat press is programmed for a 5 second pre-press and a 15 second cure press

- Verify that there is fluid in both jugs. If fluid level is below quarter full, see NIS staff before proceeding.

- On the back of the machine, ensure that the distilled water line (2) is switched on and the pre-treat line (1) is turned off.

- Open the front compartment.

- Hold a cup under the nozzle and push the prime button until the line runs completely clear.

Nozzle

Clean Line

- Once clear, switch the distilled water line to the off position, and switch the pretreat line to the on position.

- Hold a cup under the nozzle and push the prime button until the line runs completely white through the nozzle.

Line with pretreat

- Once the nozzle is primed, close the front compartment.

- Pull the drawer out and place your fabric on the platen.

- If applicable, adjust the spray length to cover more or less of the shirt.

*Setting the length at 16” will cover the platen completely.

- Close the drawer completely and press START.

- Carefully move your shirt to the heat press.

- Raise the heating element and swing it to the side to place your fabric on the platen.

- Return the heating element above the fabric and use the lever to press the fabric.

*The heating element must be locked in place by moving the lever completely down, not just set on the platen

Wrong: not locked	Correct: locked

- When your timer goes off, release the lock and raise the heating element off the platen, swing to the side and remove your fabric from the platen.

- When finished with all pretreating, turn off the pretreat line and turn on the distilled water line, put a cup under nozzle, push the prime button until line is clear.

- Clean off excess pretreat within the machine and on the platen with a wet paper towel. Dry completely.

## USING DIGITAL FACTORY SOFTWARE

- In Digital Factory, chose the correct tab for what type of printing you’re doing.

Precision Dot DTF is for Direct to Film. Precision Dot White is for Direct to Garment.

DO NOT USE

*Do not use the other tabs to the left.

- Within that tab, select the color of fabric you will be using.

-White Shirts will eliminate all white from your design, it will not print white ink, it will use the garment color (white) in place of white ink.

-Black Shirts will eliminate all black from your design, it will not print black ink, it will use the garment color (black) in place of black ink.

-Color Shirts will print all colors present in your design.

-White Only will print only the white underlayer, it will not print any of the color present in the design.

- Once you have selected the correct tab for your project, you can upload your image using the open button in the top left of the toolbar. Accepted file types include .jpg/.jpeg, .png

*If using DTF, the program will automatically mirror your design.

- You can adjust the placement and size of your image using the buttons in the toolbar for alignment or in the properties to the right. You can scale your image using the corners of the bounding box.

Scale

Properties

Alignment Tools

- You can import multiple images onto a page to print them all in one go. To fit them all on the same page right click on the page and click “Auto-nest all content”

- When your design is to your liking, select your image on the preview or in the active jobs list and click the print button in the toolbar.

Preview

## PRICING IN GARMENT CREATOR

- Once you’ve created your design in Digital Factory, you must upload the same design into garment creator using this button.

- Ensure your design is using the same print settings (i.e. black shirts vs white shirts, DTG vs DTF) and is the same size in both softwares. The latter can be done using layout settings.

Use the top point to rotate your design.

Click anywhere on the image and drag to adjust position.

Use the outer points to scale your design.

Ensure you’re using the correct platen size.

You can also make adjustments using the layout settings

- Next, click the calculator button to begin pricing.

- Ensure you’re using the “Epson Ink” settings as shown.

- Next, click “Estimate Job” at the bottom.

- In the results tab on the right, you will see the cost for printing the imported design ONCE.

- Take a picture of this tab or write down both the mL of ink used AND the cost.

- Repeat this process for each new design.

- If you’re nesting images in Digital Factory, you must do this process for

each individual image. You cannot nest images in Garment Creator.

## DIRECT TO GARMENT PRINTING

- Using the maintenance tab on the printer interface, run a light nozzle cleaning.

- Once the nozzle cleaning is complete, it will prompt you to print a pattern to check the nozzles, press the blue button on the printer interface to print the pattern.

*The pattern will print directly on the platen.

- Check the pattern for breaks in the lines for each nozzle column. Minor skips in lines will not affect the quality of your print.

- Clean the ink off the platen with a wet paper towel and dry completely.

- Pre-press your fabric in the heat press at 350 degrees for 5 seconds to eliminate wrinkles. If there is any lint on your fabric, use a lint roller to remove it.

- The printer may prompt you to shake the ink.

- Open the side compartment corresponding to the ink that needs shaking.

- Take out the cartridge that is shown on the screen with a yellow triangle symbol and gently tip it side to side for a few seconds.

- Return the cartridge to its spot with the text facing towards you.

- The cartridges have sensors that the printer uses to detect whether the ink cartridge has been placed in the correct spot.

- Put your garment on the platen, with the neck facing away from the printer.

- Place the frame over your fabric, ensuring there are no wrinkles or creases.

*Ensure that your garment is centered how you desire. There are markings on the frame to guide you.

- The printer will beep and show a preview of your design. When it is ready to print the blue button will light up. Press the blue button to begin printing.

- When printing is complete, carefully remove your garment from the platen with the ink facing away from you.

*The ink is wet and will smudge if touched. If it gets on your clothes, it can permanently stain.

- Place the garment on the platen of the heat press, with the ink facing up.

- CAREFULLY place a piece of parchment paper over the design, covering the ink completely, then press to cure it at 350 degrees for 15 seconds

*Failure to place the parchment paper carefully may result in the ink smudging. Once it is pressed the ink cannot be removed.

- Once complete you can peel the parchment paper off and feel the ink is completely dry.

- To fully cure your garment, turn it inside out and put it in the dryer on high heat for 20-30 minutes.

### DIRECT TO FILM PRINTING

- Turn on the heating element of the DTF curing machine. Once up to full temperature- 280 degrees- turn on the fan.

- Using the maintenance tab on the printer interface, run a light nozzle cleaning.

- Once the nozzle cleaning is complete, it will prompt you to print a pattern to check the nozzles, press the blue button on the printer interface to print the pattern.

*The pattern will print directly on the platen.

- Check the pattern for breaks in the lines for each nozzle column. Minor skips in lines will not affect the quality of your print.

- Clean the ink off the platen with a wet paper towel and dry completely.

- The printer may prompt you to shake the ink.

- Open the side compartment corresponding to the ink that needs shaking.

- Take out the cartridge that is shown on the screen and gently tip it side to side for a few seconds.

- Return the cartridge to its spot with the text facing towards you.

- The cartridges have sensors that the printer uses to detect whether the ink cartridge has been placed in the correct spot.

- Pre-press your fabric in the heat press at 350 degrees for 5 seconds to eliminate wrinkles. If there is any lint on your fabric, use a lint roller to remove it.

- Take one piece of film and place it matte side up on the platen. Secure it with 2 pieces of tape on opposite corners.

*Excess tape or using the frame to hold down the film will result in an error with the auto-height adjustments of the platen.

- Once your design has printed onto the film, carefully remove the tape and take the film off of the platen with the ink side facing away from you.

*The ink is wet and will smudge if touched. If it gets on your clothes, it can permanently stain.

- Place the film ink side up in the designated tub.

- Put on a mask and gloves before opening the bag of powder.

*This powder should not be ingested nor inhaled. Do this process in its designated area to avoid getting powder near the printer. DEFINE AREA

- Pour a small amount of DTF Curing White Powder into the sifter.

- Tap or shake the sifter to create a light dusting of powder over the entire design.

- Place the film ink side up onto the bed of the DTF curing machine and lower the lid.

- Press the red button to start the 60 second timer.

- Once the timer goes off, lift the lid to ensure all the powder has melted. It should look crystalline and mostly clear. If not, cure more in 5 second intervals.

- With your fabric on the platen of the heat press, swing the arm away for ample work area.

- Place the film ink side down in the desired position on your fabric.

- Cover with parchment paper and cure at 350 degrees for 15 seconds.

- Once cured, swing the arm of the heat press out of the way and remove the parchment paper.

- Peel the film from your fabric in a steady, controlled, quick manner-similar to flipping a page in a book.

- To fully cure your garment, turn it inside out and put it in the dryer on high heat for 20-30 minutes.

- Once finished, pour all excess powder from the tub back into the bag. Secure the bag shut and return the powder and sifter to the tub. Return the lid on the tub.

# HATS

- Hats can only be done with DTF. The process of printing and preparing the film for the hat is the same as normal film printing.

- Turn on the heat press specifically made for hats. This press operates the same as the regular heat press. The switch is located on the back of the interface.

  - DO NOT adjust any settings on the heat press. It is programmed for a 5 second pre-press and a 20 second cure press at 350 degrees.

Power switch

- Put the hat on the press with the bill facing up and towards you.

  - There are 3 different platen sizes. Pick the one that best fits the curved form of your hat.

- Secure the hat on the platen by putting the back strap of the hat under the half-moon form below the platen. Adjust the strap as tight as it will go around the half-moon form.

  - If your hat does not have a strap, pull as much material of the hat under the half-moon form as you can. Ensure the top sits flush with the curve of the platen.

- Pre-press your hat for 5 seconds to eliminate wrinkles or creases. If there is lint, use a lint roller to remove it.

- Place your design on the hat as desired.

  - Leave at least ¼” of space above the bill or lip of the hat.

¼” between design and bill.

- Place a piece of parchment paper over the film.

- Cure the hat by pressing it for 20 seconds at 350 degrees.

- Lift the press and remove the parchment paper.

- With your hat ON THE PLATEN, carefully remove the film; do so in a steady, controlled, quick manner- similar to flipping a page in a book.

# SPECIAL CIRCUMSTANCES

- If you are unsure if you need to pretreat a colored shirt, pretreat it just in case.

- When printing white on black it may not be fully opaque. To combat this, run your design on “White Only” first.

  - It MUST be in the exact same placement, position, and size. Send the “White Only” print first, then send the “Black Shirt” or “Color Shirt” print without moving the fabric on the platen.

  - When using this technique, if you have black in your design you should use the “Color Shirts” setting so the black color shows through the extra layer of ink.

White Only and Color Shirt print.

Only Color Shirt/Black Shirt print.

- The auto-height adjustment for the platen height is very sensitive. It can be set off and pause/restart your print for a list of reasons that include but are not limited to:

  - There is a seam on the platen that sits higher than the rest of the fabric. If possible, adjust the garment so that no seams sit on the platen.

  - A wrinkle or crease in the garment sits higher than the rest of the fabric. Smooth your fabric out with the puck sitting below the platen or by pulling on the edges of your fabric from under the frame to create more tension, flattening the fabric.

  - When using tape, if any piece is not fully secured and smoothed to the platen and starts to come up. Ensure that any tape used is completely secured to the fabric and/or platen and no piece sits higher off the platen than the fabric.