# SOP

# CNC X-CARVE PRO

## OPERATION MANUAL

Training is required before using this equipment Eye Protection Required

# MATERIAL LIMITATIONS

## ALLOWED MATERIALS

- Plywood

- Hardwoods

- Softwoods

- MDF

- HDPE

- Delrin

- Polycarbonate

- Acrylic

- Corian

METALS ARE STRICTLY PROHIBITED FROM BEING CUT ON THE XCARVE PRO

## SIZE LIMITATIONS

- Your material must be 48” x 48” or smaller

# CALIPERING MATERIAL

- Grab calipers from the peg board.

- Open the box and turn the calipers on.

- Open the calipers and slide jaws over your material.

- Close the jaws and read the measurement.

  - This EXACT measurement will be used in Easel.

# SECURING MATERIAL

YOU MUST USE AT LEAST THREE CLAMPS TO SECURE YOUR MATERIAL

- Place material onto XCarve Pro.

- Open the second drawer of the cabinet.

- Grab the metal clamps from the drawer.

- Place the bolt onto the rail system of the XCarve Pro

- Slide the clamp over to your material.

Long end

Tightening Nut

Place the longer end of the clamp over your material.

- Tighten the nut to secure the clamp to your material.

- Repeat these steps for all clamps being used.

ENSURE THE BIT WILL NOT MAKE CONTACT WITH THE METAL CLAMPS WHILE CUTTING

# TURNING MACHINE ON

- Ensure the control panel cable is connected to the computer.

- Twist the Emergency Stop on the control panel to power on.

Emergency Stop

Control Cable

# OPENING YOUR EASLE FILE

- Open Google Chrome.

- Type in https://easel.inventables.com/users/sign_in

  - Username: Innovationstudio@unl.edu

  - Password: 2021Transformation

- Locate your Easel file.

  - All files are saved to the cloud on the studio account.

# INSTALLING BIT

## NIS DOES NOT PROVIDE BITS

- Open the second drawer of the cabinet.

- Grab the two wrenches from the drawer.

  - 17mm and 22mm

17mm

22mm

- Locate the red bucket on the peg board and grab the correct collet size for your router bit.

- Insert the collet into the spindle.

- Place your bit into the collet and hand tighten.

  - Ensure bit is placed in correctly.

Bit too far out

Bit too far in

Bit Correct

- Place the 17 mm wrench on the top of the spindle.

- Place the 22 mm wrench on the bottom of the spindle.

- Push both wrenches away from yourself to tighten.

  - The bit must be very tight!

- Return both wrenches to the second drawer.

# MACHINE SET UP

- Ensure your design is finished and is ready to cut

BEFORE starting your reservation for the XCarve.

- Click on the ‘Carve’ icon in the top right corner.

- Click on the ‘Home Machine’ icon.

- Ensure that your material thickness on Easel matches your calipered material thickness EXACTLY.

- Select the closest material type to what you are cutting.

- Click Confirm Material Thickness

- Ensure material is properly secured and click the

‘Material is Secure’ icon.

- Ensure the router bit is properly installed and selected.

  - If your bit is not listed select the closest one, you will have to change the feed rate and spindle speed later.

- Click the ‘Confirm Bit Size’ icon.

# SETTING Z-AXIS ZERO

- Click the ‘Probe’ icon.

- Jog the machine using the arrow keys on the browser window.

  - Ensure bit is fully over your material.

- Click the ‘Confirm Position’ icon.

- Open the second drawer of the cabinet.

- Grab the Z-Axis Zero Plate.

- Plug the lead into the control box.

- Clip the alligator clip onto the spindle and click the ‘Clip is Attached’ icon.

- Place the metal touch plate under your bit.

- Touch the plate to your bit.

- Place the metal touch plate under your bit and click the

‘Touch Plate is in Place’ icon.

- Click the ‘Start Probing’ icon.

- After your Z-Axis is calibrated remove the alligator clip from the spindle and the plug from the control box.

- Click the ‘Z-Probe is Put Away’ icon when everything is

disconnected and placed back into the second drawer.

# SETTING X-Y AXIS ZEROS

- Jog the machine using the arrow icons on the right panel.

- Jog the machine to the bottom left corner of the machine.

- Click the ‘Set XY Zero’ icon.

# INSTALLING DUST BOOT

- Click the 'Raise to Attach Dust Boot’ icon.

- Open the second drawer of the cabinet.

- Locate and grab the dust boot.

- Ensure Silver lever is in the full up position.

- Slide the dust boot under your bit.

- Place the dust boot shaft into the slot below the silver lever.

- Push the silver lever into the full down position to lock the dust boot in place.

- Loosen the nut on the right side of the dust boot to change the height of the dust boot.

Adjustment Nut

Silver Lever

- Lower the dust boot until the bristles are making contact with your material.

- Attach dust collection tube to the dust boot.

# START CUTTING

- Click the ‘Turn on the Spindle’ icon to start the spindle.

- Ensure Spindle is on and spinning properly.

- Click the ‘The Spindle is On’ icon.

- Click the ‘CARVE!’ icon when you are ready to start

cutting.

YOU MUST BE IN THE CNC ROOM WATCHING THE XCARVE AT ALL TIMES WHILE IT IS RUNNING

# UNINSTALLING DUST BOOT

- Disconnect the dust collection tube from the dust boot.

- Raise the silver lever to the full up position.

- Pull the dust boot out of the locking slot.

- Slide the dust boot under the bit.

- Place the dust boot back into the second drawer of the cabinet.

# UNINSTALLING BIT

- Open the second drawer of the cabinet.

- Grab the two wrenches from the drawer.

  - 17mm and 22mm

17mm

22mm

- Place the 17 mm wrench on the top of the spindle.

- Place the 22 mm wrench on the bottom of the spindle.

- Pull both wrenches toward yourself to pop loose.

- Continue to loosen by hand until reaching another tight spot.

- Use wrenches again to pop the bit loose fully.

- Remove your bit and the collet completely from the spindle.

- Return both wrenches to the second drawer.

- Return the collet to the red bucket on the peg board.

# STATION CLEANUP

- Remove all clamps from your material.

- Return all clamps to the second drawer of the cabinet.

- Press the red Emergency Stop button on the control panel to turn the XCarve Pro off.